Packet Tracer Lab Guide

Configuring Wireless Security & Router Hardening

| Activity Focus | Wireless Router Hardening, Dual-SSID Segmentation (Home & Guest), IoT & Client Association, Network Isolation |
| --- | --- |
| Simulation Environment | Cisco Packet Tracer (Physical Mode locked; Logical Mode disabled) |
| Default Router Address | 192.168.0.1/24 (Default Credentials: admin / admin) |
| Validation & Grading | Auto-graded activity. Use 'Check Results' at any milestone to inspect Assessment Items. |

# Part 1: Configure Basic Security Settings for the Wireless Router

In this section, you access the graphical management interface of the Home Wireless Router to eliminate default administrative credentials and disable exposure to external management attacks.

Step 1: Change the Default Router Password

a. From the physical workspace, navigate to Home Office PC > Desktop tab > Web Browser.

b. Enter the router's management URL: http://192.168.0.1 and press Go.

c. Log in using the factory default credentials: Username: admin | Password: admin.

d. Navigate to the Administration tab.

e. Under Router Password and Re-enter to confirm, enter: cisconetacadrocks!

f. Scroll down to the bottom of the page and click Save Settings.

g. When prompted to re-authenticate, log in with Username: admin and Password: cisconetacadrocks!, then click Continue.

Step 2: Disable Remote Web Management

a. From the Administration tab, locate the Remote Management section.

b. Select the Disabled radio button.

c. Click Save Settings at the bottom of the page.

Critical Operational Procedure: Router Reset Convergence Notice: Disabling Remote Management forces the Home Wireless Router to reset its services. 1. Close the Web Browser on Home Office PC. 2. Click Fast Forward Time (Alt + D) in Packet Tracer several times to accelerate convergence. 3. Open Desktop tab > IP Configuration on Home Office PC. If the PC holds a 169.254.x.x APIPA address, toggle between Static and DHCP until it re-acquires a valid 192.168.0.x IP address with gateway 192.168.0.1. 4. Return to Web Browser, browse to 192.168.0.1, and log back in to proceed with Part 2.

# Part 2: Configure Wireless Network Profiles and Security

Configure enterprise-standard wireless parameters across all radios (2.4 GHz and 5 GHz bands), establishing dedicated production and guest network segmentation.

Step 1: Configure and Broadcast the HomeNet SSID

a. Click the Wireless tab (Basic Wireless Settings sub-menu).

b. For all three network radios (2.4GHz, 5GHz 1, and 5GHz 2): change the Network Name (SSID) from default to HomeNet.

c. Ensure SSID Broadcast is set to Enabled for all three radios (hiding SSIDs offers no true security and impedes usability).

d. Click Save Settings at the bottom of the page.

Step 2: Configure Wireless Security for HomeNet

a. From the Wireless tab, click the Wireless Security sub-menu.

b. Configure security parameters identically for all three network bands as defined in the table below:

| Parameter | Configuration Value | Security Significance |
| --- | --- | --- |
| Security Mode | WPA2 Personal | Enforces robust AES-CCMP encryption standard |
| Encryption | AES | Advanced Encryption Standard (avoids deprecated TKIP) |
| Passphrase | ciscorocks | Pre-shared key (PSK) required for client association |

c. Click Save Settings at the bottom of the page.

Step 3: Configure Guest Wireless Network Profile (GuestNet)

a. From the Wireless tab, click the Guest Network sub-menu.

b. For each of the three network radios, click the radio button to Enable Guest Profile.

c. Apply the guest configuration parameters defined in the table below across all radios:

| Parameter | Configuration Value | Operational Scope |
| --- | --- | --- |
| Guest Profile | Enabled | Activates secondary guest VAP / profile |
| SSID | GuestNet | Dedicated service identifier for visitors |
| Broadcast SSID | Enabled | Allows guest discovery |
| Security Mode | WPA2 Personal | Protects over-the-air communication |
| Encryption | AES | Strong encryption cipher |
| Passphrase | guestpass | Distinct password from internal home network |

d. Click Save Settings at the bottom of the page.

# Part 3: Configure Wireless Clients (Laptops & IoT Devices)

Associate personal workstations and smart home Internet of Things (IoT) sensors with their appropriate wireless profiles.

Step 1: Associate Client Laptops

a. Configure Home Laptop 1 (Living Room):

. • Click Home Laptop 1 > Desktop tab > PC Wireless > Connect tab.

. • Locate and select HomeNet from the available list, then click Connect.

. • Pre-shared Key: Enter ciscorocks and click Connect.

. • Verify Link Information tab displays: 'You have successfully connected to the access point'.

. • Close PC Wireless, open Desktop > IP Configuration, and verify it receives a valid 192.168.0.x IP via DHCP (toggle Static/DHCP if 169.254.x.x is shown).

b. Configure Home Laptop 2 (Home Office):

. • Click Home Laptop 2 > Desktop tab > PC Wireless > Connect tab.

. • Select GuestNet, then click Connect.

. • Pre-shared Key: Enter guestpass and click Connect.

. • Verify association on the Link Information tab and confirm DHCP assignment in IP Configuration.

Step 2: Associate Smart IoT Devices

a. Configure Home_Webcam (Home Office bookcase top shelf):

. • Click Home_Webcam > Config tab > Wireless0 interface.

. • SSID: Change to HomeNet.

. • Authentication: Select WPA2-PSK.

. • PSK Pass Phrase: Enter ciscorocks.

. • Under IP Configuration: Select DHCP and verify valid 192.168.0.x leasing.

b. Configure Home_Siren (Living Room above bookcase):

. • Repeat the exact configuration above: Config > Wireless0 > SSID: HomeNet > WPA2-PSK > ciscorocks > DHCP.

c. Configure Home Doors:

. • Repeat the exact configuration above: Config > Wireless0 > SSID: HomeNet > WPA2-PSK > ciscorocks > DHCP.

# Part 4: Verify Connectivity and Enforce Security Isolation

Validate end-to-end WAN routing and test guest wireless isolation to ensure visitors cannot reach internal household assets.

Step 1: Test Internet / WAN Connectivity

a. From Home Laptop 1, navigate to Desktop > Web Browser.

b. In the URL field, navigate to: www.ptsecurity.com and click Go.

c. If the page does not load immediately, click Fast Forward Time (Alt + D) until the Data Center Public Web Page renders.

d. From Home Laptop 2, open Desktop > Web Browser, navigate to www.ptsecurity.com, and verify successful page loading.

Step 2: Test Inter-Network Connectivity & Enforce Guest Network Isolation

a. Determine IP Address of Home Laptop 1 (e.g., check Desktop > IP Configuration or run ipconfig in Command Prompt). Note the assigned IP (e.g., 192.168.0.x).

b. From Home Laptop 2 (on GuestNet), open Desktop > Command Prompt.

c. Ping Home Laptop 1: ping <Home_Laptop_1_IP>.

d. Observation: The ping replies succeed! By default, the router bridges guest and private networks, posing an active security vulnerability.

e. Harden Guest Isolation on Router:

. • From Home Office PC, open Web Browser and connect to http://192.168.0.1.

. • Navigate to Wireless tab > Guest Network sub-menu.

. • Locate: 'Allow guests to see each other and access the local network'.

. • UNCHECK / DESELECT this checkbox.

. • Click Save Settings.

f. Re-test Ping from Home Laptop 2:

. • Return to Home Laptop 2 > Command Prompt and run ping <Home_Laptop_1_IP> again.

. • Verification: The pings now FAIL (Request timed out). Guest isolation is actively enforced!

g. Final Validation: Click the Check Results button in Packet Tracer to verify 100% completion.

# Part 5: Master Verification & Credentials Reference

| Component / Setting | Configured Value | Target Device / Verification |
| --- | --- | --- |
| Router Management URL | http://192.168.0.1 | Home Office PC Web Browser |
| Router Admin Credentials | admin / cisconetacadrocks! | Administration tab (Hardened) |
| Remote Management | Disabled | Administration tab |
| Internal SSID | HomeNet | SSID Broadcast: Enabled |
| HomeNet Security | WPA2-Personal / AES / ciscorocks | Home Laptop 1, Webcam, Siren, Doors |
| Guest SSID | GuestNet | SSID Broadcast: Enabled |
| GuestNet Security | WPA2-Personal / AES / guestpass | Home Laptop 2 |
| Guest Isolation Policy | Unchecked ('Allow guests...') | Blocks guest traffic to internal LAN |
| WAN Test Target | www.ptsecurity.com | Verified on both laptops |